CS-602: COMPUTER NETWORKS

B.Tech VI Semester -- RGPV University

DETAILED QUESTION-ANSWER BANK (HINGLISH)

Papers: May 2022 | May 2023 | May 2024 | Dec 2024 | Jun 2025

Har question ka exhaustive, in-depth Hinglish mein answer | Diagrams described | Numericals solved | Examples included

UNIT 1: NETWORK MODELS & BASICS

## Q1. OSI Reference Model kya hai? Sabhi 7 layers ka detail mein working explain karo.

Exam mein Poocha Gaya: May 2022 Q1a | May 2023 Q1b | May 2024 Q1b | Jun 2025 Q1a -- BAHUT IMPORTANT

### OSI Model ka Introduction

OSI ka full form hai Open Systems Interconnection. Yeh ek conceptual/theoretical model hai jo International Standards Organization (ISO) ne 1984 mein develop kiya. Iska main purpose tha ki alag-alag manufacturers ke computers aur devices ek dusre se communicate kar sakein -- chahe unka hardware ya software alag-alag ho.

OSI model basically define karta hai ki do network systems ke beech communication kaise honi chahiye. Yeh model 7 layers mein divide hai aur har layer ka ek specific role hota hai.

Yaad karne ka trick: 'All People Seem To Need Data Processing' -- Application, Presentation, Session, Transport, Network, Data Link, Physical.

### OSI Model -- 7 Layers (Top to Bottom)

Layer 7  |  APPLICATION LAYER   |  <-- User ke sabse paas

Layer 6  |  PRESENTATION LAYER  |

Layer 5  |  SESSION LAYER       |

Layer 4  |  TRANSPORT LAYER     |

Layer 3  |  NETWORK LAYER       |

Layer 2  |  DATA LINK LAYER     |

Layer 1  |  PHYSICAL LAYER      |  <-- Hardware ke sabse paas

### Layer 7: Application Layer

Yeh sabse upar wali layer hai aur directly end-user ke applications ke saath interact karti hai. Yeh layer networking services ko user applications tak pahunchati hai.

Kaam kya karta hai: Yeh layer ek interface provide karta hai jo user ki applications (jaise browser, email client) ko network access deta hai. Yeh layer actual application nahi hai, balki woh services hain jo applications use karti hain.

Protocols: HTTP/HTTPS (web browsing), FTP (file transfer), SMTP (email sending), POP3/IMAP (email receiving), DNS (domain name resolution), TELNET, SSH (remote login), SNMP (network management).

EXAMPLE: Jab aap browser mein www.google.com type karte ho -- yeh Application layer ka kaam hai jo HTTP request banata hai.

### Layer 6: Presentation Layer

Ise 'Translator' layer bhi kehte hain. Yeh layer data ko ek format se doosre format mein convert karta hai taaki sender aur receiver dono data samajh sakein.

Teen main kaam:

- Translation: Data ko machine-readable format mein convert karna (ASCII to EBCDIC)

- Encryption/Decryption: Secure transmission ke liye data encrypt karna (SSL/TLS)

- Compression: Bandwidth save karne ke liye data compress karna

Formats handled: JPEG, PNG, GIF (images), MPEG, AVI (video), MP3 (audio), ASCII, UNICODE (text).

EXAMPLE: Jab aap WhatsApp par image bhejte ho, Presentation layer us image ko compress karke bhejta hai.

### Layer 5: Session Layer

Session layer do communicating applications ke beech ek logical connection (session) establish, manage aur terminate karta hai. Yeh ek conversation ke shuru aur end ko manage karta hai.

- Session Establishment: Do endpoints ke beech session start karna

- Session Maintenance: Session ke dauran synchronization maintain karna, checkpoints set karna

- Session Termination: Kaam poora hone par session properly close karna

- Dialog Control: Half duplex ya full duplex communication

Protocols: NetBIOS, PPTP, RPC (Remote Procedure Call), NFS.

EXAMPLE: Ek online video call mein, session layer connection ko active rakhta hai. Agar connection temporarily toot jaaye, toh dobara resume kar sakta hai.

### Layer 4: Transport Layer

Ise 'Heart of OSI model' bhi kehte hain. Yeh layer end-to-end communication provide karta hai -- matlab source host se destination host tak reliable data delivery.

- Segmentation: Application layer se aaye bade data ko chhote segments mein todna

- Reassembly: Receiver side par segments ko wapas original data mein jodna

- Error Control: Transmission errors detect aur correct karna

- Flow Control: Sender ki speed ko receiver ki processing speed ke according control karna

- Connection Control: Connection-oriented (TCP) ya connectionless (UDP) services

- Multiplexing: Ek network connection par multiple applications data bhej sakti hain

Protocols: TCP (Transmission Control Protocol) -- reliable, connection-oriented; UDP (User Datagram Protocol) -- fast, connectionless.

EXAMPLE: Jab aap ek badi file download karte ho, Transport layer use file ko chhote TCP segments mein tod kar transfer karta hai aur doosri side par reassemble karta hai.

### Layer 3: Network Layer

Network layer ka main kaam hai logical addressing aur routing -- matlab data packets ko source se destination tak best path se deliver karna, chahe beech mein kitne bhi networks ho.

- Logical Addressing (IP Addressing): Har device ko ek unique IP address assign karna

- Routing: Data packets ke liye best path decide karna

- Packet Forwarding: Intermediate routers se packets pass karna

- Fragmentation: Bade packets ko chhote pieces mein todna (MTU ke basis par)

Protocols aur devices: IP (IPv4, IPv6), ICMP, ARP, RARP, Routers.

EXAMPLE: Jab aap India se America ke server ko request bhejte ho, Network layer har router par routing table dekh kar best path choose karta hai.

### Layer 2: Data Link Layer

Data link layer physically connected nodes ke beech reliable data transfer ensure karta hai. Yeh raw bits ko meaningful frames mein organize karta hai aur MAC addressing use karta hai.

- Framing: Data ko frames mein package karna (header + data + trailer)

- Physical/MAC Addressing: Har NIC ka unique 48-bit MAC address use karna

- Error Detection: CRC, parity se errors find karna

- Flow Control: Data link level par flow manage karna

- Access Control (MAC sublayer): Shared medium par kab kaun transmit kare

Sub-layers: LLC (Logical Link Control) -- upper layers se interface; MAC (Media Access Control) -- physical medium access.

Protocols aur devices: Ethernet, WiFi (802.11), PPP, Switches, Bridges.

### Layer 1: Physical Layer

Physical layer sabse neeche hai aur actual physical transmission ke saath deal karta hai. Yeh define karta hai ki bits ko physical medium (cables, radio waves) par kaise transmit kiya jaaye.

- Bit Transmission: Binary data (0s and 1s) ko electrical/optical/radio signals mein convert karna

- Physical Topology: Bus, Star, Ring topology

- Transmission Mode: Simplex, Half-duplex, Full-duplex

- Data Rate: Line par bits per second ki speed define karna

Protocols aur devices: Ethernet cables (Cat5, Cat6), Fiber optic, USB, Hubs, Repeaters.

EXAMPLE: Ethernet cable par current flow karna, ya WiFi antenna se radio waves bhejana -- yeh Physical layer ka kaam hai.

### Data Flow in OSI Model

Sender side par: Application (Layer 7) se shuru. Har layer neeche jaate time apna header add karta hai (ENCAPSULATION). Physical layer se actual transmission hoti hai.

Receiver side par: Physical layer se aata hai. Har layer upar jaate time apna header remove karta hai (DECAPSULATION). Application layer final data receive karta hai.

NOTE: OSI model sirf ek reference/conceptual model hai. Real-world mein TCP/IP model use hota hai jo isse simplify karta hai.

## Q2. OSI aur TCP/IP Model ka Comparative Analysis do.

Exam mein Poocha Gaya: Dec 2024 Q1a | Jun 2025 Q1a (comparison)

### TCP/IP Model Introduction

TCP/IP model ka full form hai Transmission Control Protocol/Internet Protocol. Yeh woh actual model hai jis par internet kaam karta hai. US Department of Defense ke ARPANET project se develop hua. OSI model se pehle exist karta tha.

TCP/IP mein sirf 4 layers hain (OSI ke 7 layers ko combine karke).

### TCP/IP ke 4 Layers

>> Layer 4: Application Layer

Yeh OSI ke Application + Presentation + Session layers ka combination hai. Directly user-facing protocols yahan hain.

Protocols: HTTP, HTTPS, FTP, SMTP, POP3, IMAP, DNS, DHCP, SNMP, TELNET, SSH.

>> Layer 3: Transport Layer

OSI Transport layer ke barabar. End-to-end communication.

TCP: Reliable, connection-oriented, ordered delivery, flow control, error control.

UDP: Fast, connectionless, no guarantee, used for real-time apps.

>> Layer 2: Internet Layer

OSI Network layer ke barabar. Logical addressing aur routing.

IP (IPv4/IPv6): Logical addressing. ICMP: Error reporting. ARP: IP se MAC resolve karna.

>> Layer 1: Network Access Layer (Link Layer)

OSI ke Physical + Data Link layers ka combination. Actual physical transmission aur framing.

Ethernet, WiFi, frame format, physical medium.

### OSI vs TCP/IP -- Detailed Comparison

1. Number of Layers: OSI = 7 layers. TCP/IP = 4 layers.

2. Development: OSI model pehle theoretically develop hua tha. TCP/IP practically internet ke liye ARPANET par develop hua.

3. Usage: OSI ek reference/conceptual model hai, rarely implement hota hai. TCP/IP actually internet par use hota hai.

4. Protocol Dependency: OSI model protocol-independent hai. TCP/IP specific protocols ke liye design hua hai.

5. Reliability: OSI mein network layer bhi reliable delivery support kar sakti hai. TCP/IP mein sirf Transport layer (TCP) reliable hai.

6. Session/Presentation: OSI mein separate Session aur Presentation layers hain. TCP/IP mein yeh Application layer mein merge hain.

7. Standard: OSI ek international standard hai lekin mostly theoretical. TCP/IP de facto internet standard hai.

NOTE: Interview/Exam tip: OSI ko 'reference model' bolo aur TCP/IP ko 'practical model' ya 'internet protocol suite'.

## Q3. Connection Oriented aur Connectionless Services mein antar batao.

Exam mein Poocha Gaya: May 2022 Q1b | May 2024 Q1a | Dec 2024 Q1b | Jun 2025 Q1b

### Connection Oriented Service

Connection-oriented service telephone call ki tarah kaam karta hai. Data exchange se pehle ek dedicated connection establish karna padta hai. Teen phases hoti hain: Connection establishment, Data transfer, Connection release.

>> 3-Way Handshake (TCP)

- Client server ko SYN packet bhejta hai apna Initial Sequence Number (ISN_x) ke saath.

- Server ACK (ISN_x + 1) aur apna SYN (ISN_y) bhejta hai.

- Client server ka SYN acknowledge karta hai ACK (ISN_y + 1) bhejke. Connection established!

Characteristics:

- Reliable delivery -- data guaranteed pahunchega

- Ordered delivery -- packets sahi sequence mein milenge

- Flow control -- receiver overwhelm nahi hoga

- Error control -- corrupt packets retransmit honge

- Higher overhead -- connection setup time lagta hai

- Example: TCP, ISDN, X.25, ATM

### Connectionless Service

Connectionless service post/letter ki tarah kaam karta hai. Har packet (datagram) independently travel karta hai bina kisi pre-established connection ke. Har packet mein destination address hota hai aur independently route hota hai.

- No connection setup -- directly data bhej do

- Each packet independently routed -- alag-alag paths le sakta hai

- No delivery guarantee -- packets lost ho sakte hain

- No ordering -- packets out-of-order pahunch sakte hain

- Low overhead -- faster, less processing

- Example: UDP, IP (at network layer)

### Real-world Comparison

File Transfer (FTP): TCP (Connection-oriented) -- har byte important hai.

Video Streaming: UDP (Connectionless) -- latency kam chahiye, thoda loss okay.

DNS Query: UDP -- short request-response, speed important hai.

Email: TCP -- poora email correctly deliver hona chahiye.

NOTE: Connection-oriented = telephone call analogy. Connectionless = postal letter analogy. Exam mein yeh comparison zaroor likhna.

UNIT 2: DATA LINK LAYER

## Q4. Data Link Layer mein Error Control aur Flow Control mechanisms detail mein explain karo.

Exam mein Poocha Gaya: Dec 2024 Q2a

### Error Control

Data transmission ke dauran physical medium par noise, attenuation, interference ki wajah se bits galat ho sakte hain. Error control is problem ko handle karta hai.

>> Types of Errors

Single Bit Error: Sirf ek bit change hoti hai. Example: 1000110 -> 1000100.

Burst Error: Consecutive bits change hoti hain. Yeh zyada common hai real-world mein kyunki noise burst nature ki hoti hai.

>> Error Detection Techniques

1. PARITY BIT: Data bits ke saath ek extra parity bit add kiya jaata hai. Even parity mein total 1s ki count even honi chahiye. Limitation: Only single bit errors detect kar sakta hai.

2. CRC (Cyclic Redundancy Check): Polynomial division par based hai. Sender data ko divisor se divide karta hai, remainder (CRC) end mein attach karta hai. Receiver divide karta hai -- zero remainder = no error. CRC-32 Ethernet mein use hota hai.

3. HAMMING CODE: Sirf detect hi nahi, error correct bhi karta hai (SECDED). Parity bits positions 2^0, 2^1, 2^2 par hote hain.

>> Error Correction Techniques

ARQ (Backward Error Correction): Error detect hone par sender ko dobara bhejne ki request karo. Stop-and-Wait ARQ, Go-Back-N ARQ, Selective Repeat ARQ.

FEC (Forward Error Correction): Receiver khud error correct karta hai bina retransmission ke. Hamming code, Reed-Solomon code. Wireless networks mein use.

### Flow Control

Flow control ek mechanism hai jo ensure karta hai ki ek fast sender ek slow receiver ko overwhelm na kare.

>> 1. Stop-and-Wait Protocol

Sender ek frame bhejta hai aur receiver ke ACK ka wait karta hai. ACK milne ke baad hi agla frame bhejta hai.

Efficiency = Tt / (Tt + 2*Tp) = 1 / (1 + 2a) where a = Tp/Tt

Problem: Satellite link par propagation delay zyada hone se efficiency bahut kam ho jaati hai.

>> 2. Sliding Window Protocol

Sender ko ACK ka wait kiye bina multiple frames ek saath bhejne ki permission hoti hai. Window size W = unacknowledged frames allowed.

Efficiency = W / (1 + 2*a) [if W < 1 + 2a]

EXAMPLE: W = 4. Sender frames 0,1,2,3 bhejta hai. Jab frame 0 ka ACK aata hai, window slide ho jaati hai aur frame 4 bhej sakte hain.

## Q5. Go-Back-N aur Selective Repeat Protocols ka working principle detail mein explain karo.

Exam mein Poocha Gaya: Dec 2024 Q2b | May 2022 Q4a | May 2024 Q2

### Go-Back-N (GBN) Protocol

GBN mein sender W frames ek saath send kar sakta hai, lekin agar koi frame mein error aata hai, toh us frame se lekar baaki sab frames dobara bhejne padte hain.

>> Window Sizes

Sender window: 2^n - 1 (n = sequence number bits). Example: n=3 => max window = 7.

Receiver window: 1 (sirf in-order frames accept karta hai, out-of-order DISCARD)

>> Error Handling in GBN

- Frame i mein error detect hota hai ya frame lost ho jaati hai.

- Receiver Frame i discard karta hai aur baad ke saare frames bhi discard karta hai.

- Sender timeout par Frame i se lekar sab frames dobara bhejta hai.

EXAMPLE: Window=4. Frames 0,1,2,3,4 bheje. Frame 2 lost. Frames 3,4 discard. Frames 2,3,4 dobara bheje. Inefficient!

### Selective Repeat (SR) Protocol

SR mein sirf wahi specific frames dobara bheje jaate hain jinmein actually error tha. Baaki frames waste nahi hote.

>> Window Sizes

Sender window: 2^(n-1). Example: n=3 => window = 4.

Receiver window: 2^(n-1) -- out-of-order frames buffer mein store karta hai.

>> Error Handling in SR

- Frame i mein error detect hota hai.

- Baaki frames (i+1, i+2...) jo theek hain unhe buffer mein store kar leta hai.

- Sirf Frame i dobara bheja jaata hai.

- Jab Frame i correctly receive hota hai, buffer se saare frames process karta hai.

EXAMPLE: Window=4. Frame 1 lost. Frames 2,3 buffer mein. Sirf Frame 1 dobara. Phir sab process. Efficient!

### Comparison: GBN vs SR

Sender Window: GBN = 2^n - 1. SR = 2^(n-1).

Receiver Window: GBN = 1. SR = 2^(n-1).

Retransmission: GBN = error + all subsequent. SR = only error frame.

Efficiency: SR > GBN > Stop-and-Wait.

Complexity: GBN = simpler. SR = more complex but more efficient.

NOTE: Exam mein numerical aaye: GBN window = 2^n - 1, SR window = 2^(n-1). Yeh formula important hai.

## Q6. Sliding Window Flow Control mechanism kya hai? Example se explain karo.

Exam mein Poocha Gaya: May 2023 Q2b

### Sliding Window Concept

Socho ek train platform par boxes load ho rahi hain. Platform par sirf W boxes ek saath rakh sakte hain. Jab ek box train mein load ho jaati hai (ACK milta hai), platform par ek nayi jagah ban jaati hai (window slide hoti hai).

### Detailed Example (n=3, Window=4)

Sequence numbers: 0, 1, 2, 3, 4, 5, 6, 7 (repeating).

Time 0: Window = [0,1,2,3]. Sender 0,1,2,3 bhejta hai.

Time 1: ACK 0 aata hai. Window slides to [1,2,3,4]. Frame 4 bheja.

Time 2: ACK 1 aata hai. Window = [2,3,4,5]. Frame 5 bheja.

Optimal window size: W >= 1 + 2*a for 100% efficiency (a = Tp/Tt).

## Q7. Stop-and-Wait Flow Control technique describe karo.

Exam mein Poocha Gaya: May 2022 Q3a

### Stop-and-Wait -- Complete Description

Sender ek frame bhejta hai, aur phir ACK receive karne ka wait karta hai before next frame send karna.

>> Normal Operation

- Sender Frame 0 bhejta hai.

- Receiver Frame 0 receive karta hai, ACK 0 bhejta hai.

- Sender ACK 0 receive karta hai, Frame 1 bhejta hai.

- Process repeat hoti hai.

>> Efficiency Calculation

Efficiency = Tt / (Tt + 2*Tp) = 1 / (1 + 2a)

Jahan Tt = Frame size / Bandwidth, Tp = Distance / Propagation speed

EXAMPLE: Satellite link: Tp = 270 ms, Tt = 1 ms. a = 270. Efficiency = 1/(1+540) = 0.18%. Bahut inefficient!

NOTE: Isliye satellite links par Sliding Window protocols use karte hain, Stop-and-Wait nahi.

## Q8. Bit aur Byte Stuffing kya hai? Example ke saath explain karo.

Exam mein Poocha Gaya: May 2022 Q4b

### Why Stuffing?

Data link layer data ko frames mein bhejta hai. Agar wahi delimiter sequence data mein bhi aaye, receiver galat soche ki frame khatam ho gayi. Stuffing se yeh confusion avoid hota hai.

### Byte Stuffing

Character-oriented protocols mein use hota hai. FLAG byte (0x7E) frame boundary mark karta hai. ESC byte (0x7D) bhi define hota hai.

- Agar data mein FLAG byte aaye: ESC + FLAG (0x7D + 0x7E) bhejo

- Agar data mein ESC byte aaye: ESC + ESC (0x7D + 0x7D) bhejo

EXAMPLE: Original: A FLAG B ESC C. After stuffing: FLAG A ESC FLAG B ESC ESC C FLAG.

### Bit Stuffing

Bit-oriented protocols (HDLC) mein use hota hai. FLAG = 01111110. Rule: Agar data mein 5 consecutive 1s aayein, ek 0 bit stuff karo. Receiver 5 ones ke baad 0 dekhe toh remove kar deta hai.

EXAMPLE: Original: 0 1 1 1 1 1 1 0. After stuffing: 0 1 1 1 1 1 [0] 1 0. [0] = stuffed bit.

## Q9. Data Link Layer kya hai aur yeh kaunsi services provide karta hai?

Exam mein Poocha Gaya: May 2023 Q2a

### Data Link Layer -- Services

>> Service 1: Framing

Network layer se aaye packets ko frames mein encapsulate karta hai. Frame mein: Header (MAC addresses), Data (payload), Trailer (CRC).

>> Service 2: Physical (MAC) Addressing

Har NIC ka globally unique 48-bit MAC address hota hai. Format: XX:XX:XX:XX:XX:XX (hexadecimal). MAC address same network (LAN) ke bheetar communication ke liye.

>> Service 3: Error Detection & Correction

CRC use karke frame mein errors detect karta hai. CRC mismatch par frame discard aur ARQ se retransmission.

>> Service 4: Flow Control

Sender ki speed receiver ke according adjust karna. Stop-and-Wait ya Sliding Window.

>> Service 5: Access Control (MAC Sublayer)

Multiple devices shared medium par: kab kaun transmit karega decide karna. ALOHA, CSMA/CD, CSMA/CA, Token Ring.

UNIT 3: MAC LAYER & MULTIPLE ACCESS

## Q10. Pure ALOHA aur Slotted ALOHA kya hai? Slotted ALOHA ki efficiency Pure ALOHA se double kyun hai?

Exam mein Poocha Gaya: May 2023 Q4a | May 2024 Q7b | May 2022 Q2b | Jun 2025 Q4a | Dec 2024 Q8b

### Pure ALOHA

Pure ALOHA 1970 mein University of Hawaii mein develop hua tha. Rule: Jab bhi data hai, turant transmit kar do. Koi waiting nahi.

Collision hone par: stations randomly wait karti hain (random backoff) phir dobara transmit.

Vulnerable period: 2T (do stations T pehle se lekar T baad tak kabhi bhi transmit karein, collision ho sakti hai)

Throughput: S = G * e^(-2G)

Maximum: S_max = 0.5 * e^(-1) = 18.4% (at G = 0.5)

### Slotted ALOHA

Time ko equal time slots mein divide kiya (slot size = T). Station sirf slot ki shuruat mein transmit kar sakti hai.

Vulnerable period: Sirf T (ek slot) -- synchronized transmissions

Throughput: S = G * e^(-G)

Maximum: S_max = 1 * e^(-1) = 36.8% (at G = 1)

### Why Double Throughput?

Pure ALOHA vulnerable period = 2T

Slotted ALOHA vulnerable period = T (synchronized, half the window)

Vulnerable period half hone se collision probability half ho jaati hai. Collision half = successful transmission double. Isliye exactly double throughput: 18.4% vs 36.8%.

>> Numerical Example (May 2023 Q4b)

N stations share 56 Kbps pure ALOHA channel. Each station: 1000-bit frame avg every 100 sec. Find max N.

G_per_station = (1/100) * (1000/56000) = 0.000179 frames/slot

At max throughput: G = 0.5 => N * 0.000179 = 0.5 => N = 2793 stations

## Q11. CSMA/CD Protocol ko detail mein explain karo.

Exam mein Poocha Gaya: May 2023 Q8ii | Jun 2025 Q4b

### CSMA/CD Introduction

CSMA/CD = Carrier Sense Multiple Access with Collision Detection. Ethernet (IEEE 802.3) mein use hota hai. Principle: 'Listen before talk, listen while talk.'

### Working Steps

- CARRIER SENSE: Channel monitor karo. Idle hai toh transmit karo. Busy hai toh wait karo.

- TRANSMIT: Channel idle milne par frame transmit karna shuru karo.

- COLLISION DETECTION: Transmission ke dauran continuously monitor karo. Collision detect ho toh stop.

- JAM SIGNAL: 48-bit JAM signal bhejo -- sabko collision ka pata chale.

- BACKOFF: Binary Exponential Backoff (BEB) se random time wait karo.

- RETRY: 16 attempts ke baad bhi collision hो toh error report karo.

### Binary Exponential Backoff (BEB)

After k-th collision: r = random from {0, 1, ..., min(2^k, 1024) - 1}

Wait time = r * 51.2 microseconds (Ethernet slot time)

- 1st collision: r from {0, 1}

- 2nd collision: r from {0, 1, 2, 3}

- 10th+: r from {0 to 1023} (capped)

- 16th: Give up, error

### Why CSMA/CD for Wired but not Wireless?

Wired mein collision detect karna possible hai. Wireless mein Hidden Node problem -- station doosre ki transmission nahi sun sakta. Isliye wireless mein CSMA/CA use hota hai.

## Q12. CSMA/CA mein collision se bachne ki strategies explain karo.

Exam mein Poocha Gaya: Jun 2025 Q3b | May 2024 Q8iii

### CSMA/CA -- IEEE 802.11 WiFi

Collision Avoidance kyunki wireless mein collision detection possible nahi. Hidden node problem bhi hai.

>> Strategy 1: IFS (Inter-Frame Space)

SIFS (Short IFS): High priority -- ACK, CTS. ~16 microseconds.

DIFS (Distributed IFS): Normal data frames. ~34 microseconds.

>> Strategy 2: Contention Window & Random Backoff

- Random backoff timer set karo (from Contention Window CW).

- Channel idle ho toh timer countdown.

- Channel busy ho toh timer freeze.

- Timer zero par transmit karo.

- Collision pe CW double karo.

>> Strategy 3: RTS/CTS (Request to Send / Clear to Send)

- Sender RTS frame bhejta hai (packet size, duration).

- Receiver CTS frame broadcast karta hai.

- CTS sunn ke saare stations (hidden nodes bhi) silent ho jaate hain.

- Sender data transmit karta hai.

- Receiver ACK bhejta hai.

>> Strategy 4: Mandatory ACK

Wireless mein delivery confirm karna mandatory hai. Receiver har frame par SIFS ke baad ACK bhejta hai. No ACK = retransmit.

## Q13. Binary Exponential Backoff (BEB) explain karo.

Exam mein Poocha Gaya: Dec 2024 Q4a

### BEB Purpose

Collision ke baad stations ko random time wait karke retry karna padta hai. Fixed wait hone par dobara collision hogi. BEB ensure karta hai ki baar baar collision karne par stations zyada spread out ho jaayein.

### Detailed Example

Station A aur B ek saath transmit karte hain (collision):

Collision 1 (k=1): A choose r_A from {0, 1}. B choose from {0,1}. Agar different choose kiya, no collision!

Collision 2 (k=2): Both choose from {0,1,2,3}. Lower chance of same choice.

Collision 10 (k=10): Both choose from {0 to 1023}. Very low collision probability.

After k=10, range caps at 1023 -- that's why it's 'Truncated' BEB.

NOTE: BEB is also called Truncated Binary Exponential Backoff. Caps at k=10 (range 0-1023).

## Q14. Persistent aur Non-persistent CSMA protocols explain karo.

Exam mein Poocha Gaya: Dec 2024 Q4b

>> 1-Persistent CSMA

Channel idle hone par immediately transmit karo (probability = 1). Busy ho toh continuously sense karo, idle milte hi transmit.

- Advantage: No unnecessary waiting

- Disadvantage: High collision risk -- sabhi waiting stations simultaneously transmit karte hain

>> Non-Persistent CSMA

Channel busy ho toh random time wait karo, phir dobara sense karo.

- Advantage: Fewer collisions

- Disadvantage: Channel idle reh sakta hai even when stations have data

>> p-Persistent CSMA

Slotted channel. Channel idle ho toh probability p se transmit karo, (1-p) se ek slot wait karo.

- p=1: 1-persistent behavior

- p = 1/N (N = stations): Optimal for reducing collisions

Channel utilization: 1-persistent best at light load. Non-persistent better at heavy load.

## Q15. Finite State Machine (FSM) kya hai? Network protocols mein use?

Exam mein Poocha Gaya: May 2023 Q3b | Dec 2024 Q8c

### FSM -- Complete

FSM 5 components se define hoti hai: Q (states), Sigma (inputs), delta (transitions), q0 (initial state), F (accepting states). Ek time par exactly ek state mein hoti hai.

State: System ki current condition. Example: TCP ESTABLISHED.

Event/Input: Kuch hua jo state change trigger kare. Example: SYN received.

Transition: Ek state se doosre state mein jaana.

Action: Transition ke time kuch karna. Example: SYN+ACK bhejo.

### TCP ka FSM (States)

- CLOSED: Initial state. Koi connection nahi.

- LISTEN: Server passive open, client ka wait.

- SYN_SENT: Client ne SYN bheja, server ke SYN+ACK ka wait.

- SYN_RCVD: Server ne SYN+ACK bheja, ACK ka wait.

- ESTABLISHED: Connection active. Data transfer.

- FIN_WAIT_1/2, TIME_WAIT: Active close phases.

- CLOSE_WAIT, LAST_ACK: Passive close phases.

### FSM Benefits in Protocol Design

- Protocol behavior unambiguously define hoti hai

- Testing easy -- har state-transition test karo

- Formal verification possible

- Implementation guide -- developers exact code likh sakte hain

UNIT 4: NETWORK LAYER

## Q16. IP Address classes explain karo -- Network-ID, Host-ID, ranges.

Exam mein Poocha Gaya: May 2024 Q4a | May 2022 Q3b

### IP Address Introduction

IP (IPv4) address = 32-bit logical address. Dotted decimal: W.X.Y.Z (each 0-255). Two parts: Network ID (kaun sa network) aur Host ID (us network ka kaun sa device).

### Class A -- Bahut bade Networks

First bit: 0 (fixed). Network bits = 8. Host bits = 24.

Range: 0.0.0.0 to 127.255.255.255

Networks: 126. Hosts per network: 16,777,214

Subnet mask: 255.0.0.0 (/8)

EXAMPLE: 10.0.0.0 (private). Publicly: 17.0.0.0 = Apple Inc.

### Class B -- Medium Networks

First bits: 10 (fixed). Network bits = 16. Host bits = 16.

Range: 128.0.0.0 to 191.255.255.255

Networks: 16,384. Hosts per network: 65,534

Subnet mask: 255.255.0.0 (/16)

EXAMPLE: 172.16.0.0 to 172.31.255.255 (private Class B).

### Class C -- Small Networks

First bits: 110 (fixed). Network bits = 24. Host bits = 8.

Range: 192.0.0.0 to 223.255.255.255

Networks: 2,097,152. Hosts per network: 254

Subnet mask: 255.255.255.0 (/24)

EXAMPLE: 192.168.0.0 (most common private network).

### Class D & E

Class D (224-239.x.x.x): Multicast groups. Video streaming. No host/network division.

Class E (240-255.x.x.x): Research/experimental. Not publicly routable.

### Special Addresses

127.0.0.1: Loopback (localhost) -- apne aap ko test karne ke liye.

255.255.255.255: Limited broadcast.

Network address: Host part = all 0s. Example: 192.168.1.0

Broadcast address: Host part = all 1s. Example: 192.168.1.255

NOTE: CIDR (Classless Inter-Domain Routing) ab use hota hai -- /prefix notation, fixed class boundaries nahi.

## Q17. Subnetting: 211.17.180.0/24 mein 32 subnets banao.

Exam mein Poocha Gaya: Jun 2025 Q7 | May 2024 Q5

### Subnetting Solution

>> Given Information

- Network address: 211.17.180.0, Prefix: /24, Required subnets: 32

>> Step 1: Bits to Borrow

2^n >= 32 => 2^5 = 32 => n = 5 bits borrow karo

>> Step 2: New Subnet Mask

New prefix: /24 + 5 = /29

Subnet mask: 255.255.255.248 (binary: 11111111.11111111.11111111.11111000)

>> Step 3: Hosts per Subnet

Host bits = 32 - 29 = 3 bits

Usable hosts = 2^3 - 2 = 6

>> Step 4: Block Size & Subnets

Block size = 256 - 248 = 8

1st Subnet: Network=211.17.180.0, First host=.1, Last host=.6, Broadcast=.7

2nd Subnet: Network=211.17.180.8, First host=.9, Last host=.14, Broadcast=.15

32nd Subnet: Network=211.17.180.248, First host=.249, Last host=.254, Broadcast=.255

### Summary

Subnet Mask: 255.255.255.248 (/29)

Total Subnets: 32

Usable Hosts per Subnet: 6

Block Size: 8

NOTE: Network address aur broadcast address hosts ke liye use nahi hote. Isliye 2^n - 2 usable hosts.

## Q18. ARP aur RARP ki karyapranali samjhao.

Exam mein Poocha Gaya: Dec 2024 Q3b | May 2022 Q8ii | May 2024 Q8ii

### ARP -- Complete Working

ARP (RFC 826) IP-to-MAC address resolution karta hai. Network layer IP address jaanti hai, Data Link layer MAC address chahiye frame banana ke liye.

- Sender apni ARP cache check karta hai. Agar mapping mili, use karo.

- Nahi mila toh ARP Request BROADCAST karo (dest MAC = FF:FF:FF:FF:FF:FF).

- Network par saari devices broadcast receive karti hain.

- Sirf target device ARP Reply bhejti hai (unicast).

- Sender mapping ARP cache mein store kar leta hai (typically 20 min).

Proxy ARP: Router ek doosre network ke behalf par reply karta hai.

Gratuitous ARP: Device apna khud ka IP ke liye ARP -- IP conflict check.

### RARP -- Complete Working

RARP ARP ka ulta. Device MAC address jaanti hai, IP nahi (diskless workstations, booting).

- Device RARP Request broadcast karta hai apna MAC ke saath.

- RARP Server apni table mein IP dhundhta hai aur RARP Reply bhejta hai.

- Device apna IP jaanti hai.

Limitation: Har network par RARP server chahiye. DHCP ne replace kar diya -- IP + subnet + gateway + DNS sab assign karta hai.

## Q19. Bellman-Ford Algorithm explain karo.

Exam mein Poocha Gaya: Dec 2024 Q5a | May 2022 Q8iv | Jun 2025 Q8iv

### Bellman-Ford -- Complete

Dynamic programming based shortest path algorithm. Negative edge weights bhi handle karta hai. Distance Vector routing (RIP) isi par based hai.

>> Algorithm Steps

- INITIALIZE: dist[source] = 0. dist[all others] = infinity.

- RELAX (V-1 iterations): For each edge (u,v,w): if dist[u]+w < dist[v]: dist[v] = dist[u]+w

- CHECK: Ek aur iteration -- agar koi distance update ho toh negative cycle hai.

>> Example

Nodes A-E. Edges: A-B=4, A-C=2, B-C=1, B-D=5, C-D=8, C-E=10, D-E=2. Source=A.

After iterations: dist = {A:0, B:4, C:2, D:9, E:11}

Time Complexity: O(V * E) -- Slower than Dijkstra but handles negative weights

### Count-to-Infinity Problem

Agar link fail ho jaaye, bad news dheere propagate hoti hai. Routers ek dusre ko false info dete hain incrementally badhate hue.

Solutions: Split horizon, Poisoned reverse, Hold-down timers, Max hop count (RIP = 15).

## Q20. ICMP par comprehensive note likho.

Exam mein Poocha Gaya: May 2023 Q5b | May 2022 Q8iii | Jun 2025 Q8iii

### ICMP Introduction

ICMP (Internet Control Message Protocol) RFC 792. IP ka companion -- IP unreliable hai, ICMP error messages aur diagnostic info source ko bhejta hai. ICMP messages khud IP packets mein encapsulate hote hain.

### Important ICMP Types

>> Type 0 & 8: Echo Reply & Echo Request (PING)

Ping ICMP Echo Request (Type 8) bhejta hai. Target Echo Reply (Type 0) se respond karta hai. RTT measure hota hai.

EXAMPLE: ping google.com => Echo Request to Google IP, Reply received. RTT = 20ms.

>> Type 3: Destination Unreachable

- Code 0: Network Unreachable

- Code 1: Host Unreachable

- Code 2: Protocol Unreachable

- Code 3: Port Unreachable

>> Type 11: Time Exceeded

TTL = 0 hone par router packet drop karta hai aur ICMP Time Exceeded bhejta hai. TRACEROUTE yahi use karta hai.

Traceroute: TTL=1 => first router respond. TTL=2 => second router. Continue to destination.

## Q21. Fragmentation aur Reassembly explain karo.

Exam mein Poocha Gaya: Dec 2024 Q5b | May 2023 Q8iv

### Why Fragmentation?

Har network link ka MTU hota hai (Ethernet = 1500 bytes). IP packet MTU se bada ho toh todna padta hai.

### IP Header Fields

Identification (16 bits): Sab fragments ki common ID.

Flags: DF bit: Don't Fragment -- set hone par discard karo, fragment mat karo.

Flags: MF bit: More Fragments -- 1 = aur aayenge, 0 = last fragment.

Fragment Offset (13 bits): Original data mein position (units of 8 bytes).

### Example

Packet: 20 byte header + 4000 byte data. MTU = 1500. Max data per fragment = 1480.

Fragment 1: Data 0-1479. Offset=0. MF=1. Total=1500.

Fragment 2: Data 1480-2959. Offset=185. MF=1. Total=1500.

Fragment 3: Data 2960-3999. Offset=370. MF=0. Total=1060.

NOTE: Sirf destination reassembly karta hai (IPv4). IPv6 mein sirf source fragment karta hai.

## Q22. IPv4 aur IPv6 ka comparison.

Exam mein Poocha Gaya: May 2023 Q6b

### Why IPv6?

IPv4 mein 2^32 = 4.3 billion addresses. 2011 mein almost exhaust ho gaye. IPv6 solution hai.

Address Size: IPv4 = 32 bits. IPv6 = 128 bits (2^128 addresses -- practically unlimited).

Notation: IPv4 = dotted decimal. IPv6 = colon-hex: 2001:db8::8a2e:370:7334.

Header Size: IPv4 = 20-60 bytes (variable). IPv6 = fixed 40 bytes (faster routing).

Checksum: IPv4 = header checksum. IPv6 = no header checksum (faster, transport layer handles).

Fragmentation: IPv4 = intermediate routers bhi kar sakte. IPv6 = sirf source karta hai.

Security: IPv4 = IPSec optional. IPv6 = IPSec built-in.

Auto-config: IPv4 = DHCP required. IPv6 = SLAAC (device khud IP generate kare).

Broadcast: IPv4 = broadcast exists. IPv6 = no broadcast, only multicast/anycast.

NOTE: IPv6 adoption ongoing. Many ISPs dual stack (IPv4+IPv6) support karte hain.

## Q23. Broadcast aur Multicast Routing mein difference.

Exam mein Poocha Gaya: Dec 2024 Q6b

### Unicast vs Broadcast vs Multicast

Unicast: Ek source, ek specific destination. Normal internet traffic.

### Broadcast

Ek source saare network ke nodes ko packet bhejta hai. Destination = 255.255.255.255 ya network broadcast address.

- Limited Broadcast (255.255.255.255): Sirf local network, routers cross nahi karta.

- Directed Broadcast: Specific network ki sabhi hosts. Example: 192.168.1.255.

- Use cases: ARP requests, DHCP discovery.

- Problem: Broadcast storm possible, sabhi devices interrupt hoti hain.

### Multicast

Sirf un nodes ko packet bhejta hai jo ek multicast group ke member hain. Class D address (224.0.0.0 to 239.255.255.255).

- IGMP: Hosts group join/leave karte hain.

- PIM: Routers multicast routing tree banate hain.

- Use: IPTV, online gaming, stock updates.

Advantage: Sirf interested receivers ko traffic jaata hai -- less network load.

## Q24. Binary Count Down Protocol explain karo.

Exam mein Poocha Gaya: Dec 2024 Q6a

### Binary Countdown Protocol

Har station ko binary address assign hota hai. Agar multiple stations transmit karna chahti hain, distributed arbitration hoti hai.

>> Working

- Stations apne address ke MSB se shuru karke ek-ek bit broadcast karti hain.

- Channel par wired-OR hoti hai (koi bhi 1 bheje toh channel par 1).

- Station ka bit 0 tha lekin channel par 1 dikh raha hai toh woh drop out.

- Highest address wali station survive karti hai -- wins!

- Winner apna data transmit karta hai.

EXAMPLE: Stations 001, 011, 100 compete. Bit 1 (MSB): 001=0, 011=0, 100=1. Channel=1. 001,011 drop out. Station 100 wins!

Properties: Deterministic (highest address always wins). Collision-free. O(log N) bits per arbitration.

UNIT 5: TRANSPORT LAYER

## Q25. TCP header aur UDP header ki tulna karo.

Exam mein Poocha Gaya: May 2024 Q3 | Jun 2025 Q6 | May 2022 Q8iv

### UDP Header (8 bytes only)

| Source Port (16) | Destination Port (16) |

| Length (16)      | Checksum (16)         |

Source Port: Sender application ka port. Reply ke liye.

Destination Port: Receiver application. DNS=53, DHCP=67/68, SNMP=161.

Length: Header + data. Min=8, Max=65535.

Checksum: IPv4 mein optional. IPv6 mein mandatory.

### TCP Header (20+ bytes)

| Source Port (16)       | Destination Port (16)  |

| Sequence Number (32 bits)                        |

| Acknowledgment Number (32 bits)                  |

| HL | Reserved | Flags (6) | Window (16)          |

| Checksum (16)          | Urgent Pointer (16)    |

| Options (0-40 bytes)                             |

Sequence Number (32): Byte stream mein position. Ordered delivery ke liye.

ACK Number (32): Next expected byte. Cumulative ACK.

Flags (6 bits): URG, ACK, PSH, RST, SYN, FIN -- connection control.

Window Size (16): Receiver buffer space -- flow control.

Checksum: TCP mein mandatory.

### Why Extra Fields in TCP?

Seq/ACK Numbers: Reliable ordered delivery ke liye. UDP mein ordering ki zaroorat nahi.

Window Size: TCP flow control karta hai. UDP mein koi flow control nahi.

SYN/FIN/RST: Connection-oriented hai TCP. UDP connectionless.

## Q26. TCP machine 65535B window, 1 Gbps channel, 10ms one-way delay. Throughput aur efficiency nikalo.

Exam mein Poocha Gaya: May 2024 Q6 | Jun 2025 Q5

### Given

- Window W = 65535 bytes, Bandwidth B = 1 Gbps = 10^9 bps, One-way Tp = 10 ms

### Solution

W in bits = 65535 * 8 = 524,280 bits

Transmission time Tt = 524,280 / 10^9 = 0.5243 ms

RTT = 2 * Tp = 20 ms

Throughput = W / (Tt + RTT) = 524,280 / (0.0005243 + 0.020) = 25.5 Mbps

Efficiency = Tt / (Tt + RTT) = 0.5243 / 20.5243 = 2.55%

### Interpretation

Sirf 2.55% channel capacity use ho rahi hai! Optimal window = RTT * B = 0.020 * 10^9 / 8 = 2.5 MB hona chahiye. Isliye TCP Window Scaling option use hota hai.

## Q27. TCP Connection Establishment aur Release explain karo.

Exam mein Poocha Gaya: May 2022 Q5a

### 3-Way Handshake

>> Step 1: SYN (Client to Server)

SYN flag=1. Sequence=ISN_c. State: Client -> SYN_SENT.

>> Step 2: SYN+ACK (Server to Client)

SYN=1, ACK=1. Server seq=ISN_s. ACK=ISN_c+1. State: Server -> SYN_RCVD.

>> Step 3: ACK (Client to Server)

ACK=1. ACK number=ISN_s+1. State: Both -> ESTABLISHED.

Why 3-Way (not 2)? 2-way sirf ek direction mein confirm karta hai. 3-way DONO sides ki ISN exchange aur readiness confirm karta hai.

### 4-Way Connection Release

- FIN (active closer): 'Main aur data nahi bhejunga.' State: FIN_WAIT_1.

- ACK (passive): 'FIN mila.' State: Active -> FIN_WAIT_2, Passive -> CLOSE_WAIT.

- FIN (passive): 'Main bhi done hoon.' State: Active -> TIME_WAIT.

- ACK (active): Final ACK. 2*MSL wait phir CLOSED.

TIME_WAIT (2*MSL): Ensure karna ki passive side ka FIN/ACK network mein expire ho jaaye.

## Q28. TCP Flow Control aur Congestion Control explain karo.

Exam mein Poocha Gaya: May 2023 Q7b | Jun 2025 Q8ii | Dec 2024 Q8a

### TCP Flow Control

Receiver ko overwhelm hone se protect karta hai. Receiver Window (rwnd) TCP header mein advertise hota hai.

Sender constraint: LastByteSent - LastByteACKed <= rwnd

Buffer full hone par rwnd=0 advertise -- sender ruk jaata hai. Buffer free hone par non-zero rwnd.

### TCP Congestion Control (Slow Start / Congestion Avoidance)

Effective window = min(rwnd, cwnd)  [cwnd = congestion window]

>> Phase 1: Slow Start

cwnd = 1 MSS. Har ACK par cwnd++ (doubles every RTT -- exponential). Jab cwnd = ssthresh, next phase.

>> Phase 2: Congestion Avoidance

Linear growth: cwnd += 1 MSS per RTT (additive increase). Safer network probing.

>> Phase 3: Congestion Detection

TIMEOUT: ssthresh = cwnd/2. cwnd = 1. Slow start restart.

3 Duplicate ACKs: ssthresh = cwnd/2. cwnd = ssthresh (TCP Reno Fast Recovery). Fast Retransmit.

AIMD pattern -- Additive Increase, Multiplicative Decrease -- creates sawtooth pattern.

NOTE: Modern TCP: CUBIC (Linux default), BBR (Google), QUIC (HTTP/3) more sophisticated congestion control.

## Q29. UDP header format aur features.

Exam mein Poocha Gaya: Dec 2024 Q7a

### UDP Features

- Connectionless: Koi handshake nahi

- Unreliable: No ACK, no delivery guarantee

- Unordered: Kisi bhi order mein pahunch sakte hain

- No flow/congestion control

- Low latency: No connection setup

- 8 byte header only (vs TCP 20+)

- Supports broadcast/multicast

### Use Cases

DNS: Short query-response. Speed important.

Video/Audio Streaming: Thoda loss okay, latency nahi.

Online Gaming: Real-time updates, old data useless.

VoIP: Voice calls, latency critical.

## Q30. Transport Layer QoS parameters kya hain?

Exam mein Poocha Gaya: May 2024 Q7a

>> 1. Bandwidth (Throughput)

Unit time mein data transfer. Video HD streaming = 5 Mbps+. Email = low bandwidth.

>> 2. Delay (Latency)

Packet source se destination tak. VoIP = <150ms required. File download = less critical.

>> 3. Jitter

Consecutive packets ki delay variation. Real-time audio/video mein choppy output. Solution: Jitter buffer.

>> 4. Reliability (Packet Loss)

File transfer = 0% loss (TCP). Voice = 1-2% acceptable. Video = 5% acceptable.

>> 5. Security

Authentication, Confidentiality (encryption), Integrity. Banking = high security required.

### QoS Mechanisms

- IntServ: Per-flow reservation (RSVP protocol)

- DiffServ: Packet marking (DSCP field)

- Traffic shaping: Leaky bucket, token bucket

UNIT 6: APPLICATION LAYER & SPECIAL TOPICS

## Q31. SNMP aur uske functions detail mein explain karo.

Exam mein Poocha Gaya: Dec 2024 Q7b

### SNMP Introduction

SNMP (Simple Network Management Protocol) application layer protocol. Network devices remotely monitor aur manage karne ke liye. UDP port 161/162.

### 3 Components

NMS/Manager: Central monitoring station. GET, SET queries bhejta hai.

Managed Devices + Agents: Har device par SNMP Agent. Device info collect karta hai.

MIB (Management Information Base): Har device par managed objects ka database. OID se identify.

### SNMP Operations

GET: Manager specific object ki value query karta hai.

GET-BULK (v2+): Multiple objects efficiently retrieve.

SET: Manager kisi object ki value change karta hai.

TRAP (Agent->Manager): Agent bina pooche events notify karta hai. Interface down, high temp. Unreliable (UDP).

INFORM (v2+): TRAP + ACK = reliable delivery.

### Versions

SNMPv1: Basic. Community strings (cleartext -- insecure).

SNMPv2c: GET-BULK added. Still insecure.

SNMPv3: Authentication (MD5/SHA), encryption (AES), access control. Current standard.

## Q32. Electronic Mailing kya hai? E-mail bhejne ke different tarike.

Exam mein Poocha Gaya: May 2023 Q7a

### Email Protocols

SMTP (port 25/587): Mail bhejne ke liye -- client to server aur server to server.

POP3 (port 110/995): Mail download karke local machine par store. Server se delete.

IMAP (port 143/993): Mail server par hi rakhta hai. Multiple devices access. Modern standard.

### Email Sending Process

- User mail client mein email compose karta hai.

- MUA SMTP se mail submission server (587) se connect.

- MSA email accept, sender authenticate.

- MTA recipient domain DNS mein MX record dhundhta hai.

- Sender's MTA SMTP se recipient's MTA se connect (port 25).

- Email recipient's mailbox mein store.

- Recipient POP3/IMAP se retrieve karta hai.

### Email ke Different Tarike

Webmail: Browser se directly. Gmail, Yahoo, Outlook.com.

Email Client: Outlook, Thunderbird. POP3/IMAP. Offline access.

Mobile Apps: Gmail app, IMAP + push notifications.

Command Line: Linux 'mail' command. Scripts se automated.

API-based: Sendgrid, Mailgun. Transactional emails.

## Q33. Guided aur Unguided transmission media discuss karo.

Exam mein Poocha Gaya: May 2023 Q3a

### Guided Media (Wired)

>> 1. Twisted Pair Cable

Do insulated copper wires ek dusre ke around twist. Twisting EMI aur crosstalk reduce karta hai.

- Cat5e: 1 Gbps, 100m, 100 MHz

- Cat6: 10 Gbps (55m), 250 MHz

- Cat6a: 10 Gbps at 100m, 500 MHz

Use: Telephone lines, LAN (Ethernet), DSL.

>> 2. Coaxial Cable

Central copper conductor, dielectric insulator, metallic shield, outer jacket. Better noise immunity.

- 50-ohm: Baseband digital data (Ethernet 10Base5)

- 75-ohm: Cable TV, broadband

>> 3. Fiber Optic Cable

Light signals (photons). Total internal reflection principle. Very high bandwidth (Tbps), immune to EMI.

Single-mode: Thin core (8-10 micron). Long distance (100+ km). Laser. Expensive.

Multi-mode: Wide core (50-62.5 micron). Short distance (<2 km). LED. Cheaper.

### Unguided Media (Wireless)

>> 1. Radio Waves (3 kHz - 1 GHz)

Omnidirectional. Buildings penetrate kar sakte hain. WiFi (2.4/5/6 GHz), Bluetooth, Cellular.

>> 2. Microwaves (1 GHz - 300 GHz)

Line-of-sight required. Terrestrial (tower-to-tower) aur Satellite (GEO/MEO/LEO) microwaves.

>> 3. Infrared

Very short range, LOS required, walls penetrate nahi. TV remotes, proximity sensors.

## Q34. 4 kHz bandwidth, signal 20V, noise 6mV. Maximum data rate nikalo.

Exam mein Poocha Gaya: Dec 2024 Q3a

### Solution using Shannon's Theorem

Given: B = 4 kHz = 4000 Hz, Signal = 20V, Noise = 6mV = 0.006V

>> Step 1: Calculate SNR

SNR = Signal / Noise = 20 / 0.006 = 3333.33

>> Step 2: Shannon Capacity

C = B * log2(1 + SNR) = 4000 * log2(1 + 3333.33)

C = 4000 * log2(3334.33) = 4000 * 11.7 = 46,800 bps ~= 46.8 Kbps

### Nyquist Theorem (Noiseless channel)

Max rate = 2 * B * log2(M)  [M = discrete signal levels]

NOTE: Shannon = absolute max for noisy channel. Nyquist = noiseless limit. Use Shannon when noise given.

## Q35. Different types of topologies -- advantages aur disadvantages.

Exam mein Poocha Gaya: May 2024 Q1a

>> 1. Bus Topology

Sabhi devices ek single shared cable (backbone) se connected.

- Advantages: Simple, less cable, easy to expand, cost effective.

- Disadvantages: Single point of failure (bus fail = sab fail). Collisions common. Limited length.

>> 2. Star Topology

Sabhi devices central hub/switch se connected.

- Advantages: Easy troubleshooting, one link fail = sirf that device, easy expansion.

- Disadvantages: Hub/switch = single point of failure. More cable required.

EXAMPLE: Modern Ethernet LAN, Home WiFi network.

>> 3. Ring Topology

Har device exactly 2 doosre se connected. Token passing.

- Advantages: No collisions, equal access.

- Disadvantages: One break = ring failure. Adding devices disrupts network.

>> 4. Mesh Topology

Har node har doosre se directly (full mesh) ya kuch multiple connections (partial mesh).

- Advantages: Highly reliable, multiple paths, fault tolerant.

- Disadvantages: Most expensive. n(n-1)/2 connections for full mesh.

>> 5. Tree (Hierarchical)

Star topologies hierarchy. Core switch -> distribution -> access.

- Advantages: Scalable, easy management, segment isolation.

- Disadvantages: Root failure = big disruption.

>> 6. Hybrid

Do ya zyada topologies ka combination. Most real-world networks hybrid hain.

## Q36. Frame Relay Protocol Architecture describe karo.

Exam mein Poocha Gaya: May 2022 Q5b

### Frame Relay Overview

WAN protocol, packet-switched, virtual circuits use karta hai. 1980s mein popular WAN technology.

### Key Concepts

PVC (Permanent Virtual Circuit): Manually configured permanent connection.

SVC (Switched Virtual Circuit): On-demand connection setup.

DLCI (10-bit): Virtual circuit identifier. Local significance.

### Components

DTE (Customer equipment): Routers, switches at network edge.

DCE (Carrier equipment): Frame Relay switches in cloud.

### Frame Format

Flag | Header (DLCI + flags) | Data | FCS | Flag

FECN: Forward Explicit Congestion Notification -- receiver ko notify.

BECN: Backward Explicit Congestion Notification -- sender ko slow down.

DE (Discard Eligibility): DE=1 wale frames pehle discard hote hain congestion mein.

## Q37. Distance Vector Routing explain karo.

Exam mein Poocha Gaya: May 2022 Q6b

### Distance Vector Routing

Distributed routing algorithm. Har router apne neighbors ke saath routing info share karta hai. Bellman-Ford use karta hai.

>> How it works

- Har router sirf directly connected neighbors ko jaanta hai initially.

- Periodically (RIP = every 30 sec) complete routing table neighbors ko bhejo.

- Receiver Bellman-Ford se shortest paths calculate karta hai.

- Better path milne par routing table update karo.

- Eventually (convergence) poori network correct paths jaanti hai.

>> Count to Infinity Solutions

Split Horizon: Neighbor ko woh route mat batao jo tumne usse sikh hai.

Poisoned Reverse: Route report karo with infinite distance.

Max hop count: RIP mein 15 = max, 16 = infinity.

RIP: Simple distance vector. Hop count metric. 30 sec updates.

## Q38. Routing aur Congestion Control samjhao.

Exam mein Poocha Gaya: May 2022 Q7a

### Routing

Network layer ka fundamental kaam -- source se destination tak packets forward karna. Routers routing tables maintain karte hain.

Static Routing: Manually configured. Simple, predictable. Not scalable.

Dynamic Routing: Automatic -- RIP, OSPF, BGP. Adaptive, scalable.

IGP: Ek AS ke andar -- RIP, OSPF, IS-IS.

EGP: Alag AS ke beech -- BGP (internet backbone).

### Congestion Control Mechanisms

Leaky Bucket: Traffic ek fixed rate par output. Bursty input -> steady output.

Token Bucket: Tokens at fixed rate generate. Har packet ke liye ek token consume. Burst allowed if tokens available.

RED (Random Early Detection): Buffer full hone SE PEHLE randomly packets drop. TCP connections slow down proactively.

Backpressure: Downstream se upstream congestion signal propagate.

## Q39. Dijkstra's Algorithm (Least Cost Routing) explain karo.

Exam mein Poocha Gaya: May 2023 Q5a

### Dijkstra's Algorithm

Greedy algorithm. Single-source shortest path. Non-negative weights required. OSPF, IS-IS use karte hain.

>> Steps

- INIT: dist[source]=0. dist[others]=infinity. S (visited) = empty.

- LOOP: S se bahar minimum dist wala node v choose karo.

- Add v to S.

- For each neighbor w: if dist[v]+cost(v,w) < dist[w]: update dist[w], predecessor[w]=v.

- Repeat until all nodes in S.

>> Example

Nodes A-E. Source=A. A-B=2, A-C=4, B-C=1, B-D=7, C-E=3, D-F=1, E-D=2.

Result: A=0, B=2, C=3, D=8, E=6.

Time: O(V^2) simple, O((V+E) log V) with priority queue.

### Link State vs Distance Vector

Convergence: Link State (OSPF) fast. Distance Vector (RIP) slow.

CPU/Memory: Link State more. Distance Vector less.

Scalability: Link State better (areas). Distance Vector poor.

## Q40. Error Detection kya hai? 10101111 ko even-parity Hamming Code se encode karo.

Exam mein Poocha Gaya: Jun 2025 Q3a

### Hamming Code -- Complete Encoding

>> Step 1: Number of parity bits

2^r >= m + r + 1, m = 8 data bits

r=4: 2^4=16 >= 13. So 4 parity bits. Total bits = 12.

>> Step 2: Place bits

Positions: 1  2  3  4  5  6  7  8  9  10 11 12

Type:      P1 P2 D1 P4 D2 D3 D4 P8 D5 D6 D7 D8

Data (10101111): D1=1,D2=0,D3=1,D4=0,D5=1,D6=1,D7=1,D8=1

Placed: P1 P2  1 P4  0  1  0 P8  1  1  1  1

>> Step 3: Calculate parity bits (Even parity)

P1 covers pos 1,3,5,7,9,11: bits=1,0,0,1,1. Count of 1s=3 (odd). P1=1.

P2 covers pos 2,3,6,7,10,11: bits=1,1,0,1,1. Count=4 (even). P2=0.

P4 covers pos 4,5,6,7,12: bits=0,1,0,1. Count=2 (even). P4=0.

P8 covers pos 8,9,10,11,12: bits=1,1,1,1. Count=4 (even). P8=0.

>> Step 4: Final encoded (12 bits)

Positions: 1  2  3  4  5  6  7  8  9  10 11 12

Bits:      1  0  1  0  0  1  0  0  1  1  1  1

Error detection: Agar position 6 mein error ho, P2 aur P4 fail honge. 2+4=6 => position 6 mein error. Correct karo!

## Q41. Limited-Contention Protocols -- Adaptive Tree Walk Protocol.

Exam mein Poocha Gaya: May 2023 Q6a

### Adaptive Tree Walk Protocol

Binary tree mein stations organize. Low load par ALOHA-like. Heavy load par collision-free. Best of both!

>> Algorithm

- Slot 0: Sabhi stations compete (root level). 1 frame = success. 0 = idle. Collision = go deeper.

- Collision: Left subtree slot 1 mein, Right subtree slot 2 mein compete.

- DFS traversal -- left subtree completely resolve phir right subtree.

>> Example (8 stations, 1,3,5 want to transmit)

Slot 1: All compete. Collision.

Slot 2: Left (0-3). Stations 1,3. Collision.

Slot 3: Left-left (0-1). Station 1 only. SUCCESS!

Slot 4: Left-right (2-3). Station 3 only. SUCCESS!

Slot 5: Right (4-7). Station 5 only. SUCCESS! All done in 5 slots.

Advantages: Bounded delay O(N). No wasted slots after first collision. Self-correcting under load.

## Q42. Computer Network kya hai? Importance, merits, demerits.

Exam mein Poocha Gaya: May 2023 Q1a

### Definition

Computer network = interconnected computers/devices jo resources share karte hain aur communicate karte hain. PAN, LAN, MAN, WAN -- scale vary karta hai.

### Importance

Resource Sharing: Printer, scanner, servers -- sabhi authorized users share kar sakte hain.

Communication: Email, video call, VoIP -- distance barrier nahi.

Data Sharing: Centralized databases, file sharing.

Internet Access: Global information, e-commerce, cloud services.

### Merits

- Global communication instantly

- Resource sharing -- cost reduction

- Centralized data management

- Remote work aur education possible

- Real-time data access

### Demerits

- Security risks -- hacking, data theft

- Virus/malware spread

- Network down = work stops

- Setup cost -- cables, switches, routers

- Maintenance complexity

- Privacy concerns

## Q43. FDDI kya hai? Explain.

Exam mein Poocha Gaya: May 2022 Q8i

### FDDI -- Complete

FDDI = Fiber Distributed Data Interface. ANSI/ISO standard. High-speed LAN ke liye. 1980s mein backbone networks par popular.

Speed: 100 Mbps (Ethernet sirf 10 Mbps tha -- 10x faster)

Medium: Fiber optic (FDDI) ya copper (CDDI)

Distance: Station-to-station: 2 km. Ring max: 200 km.

Topology: DUAL counter-rotating token rings (primary + secondary)

Max stations: 500 per ring

### Dual Ring Fault Tolerance

Normal: Sirf primary ring active. Secondary = standby. Agar primary ka segment fail ho toh stations 'wrap' karte hain -- primary+secondary ko ek single ring mein combine. Network continue karta hai!

### Classes

Class A: Primary + Secondary ring attached. Full fault tolerance.

Class B: Sirf Primary ring. Single attachment.

NOTE: FDDI aaj outdated -- Gigabit Ethernet ne replace kar diya. But exam mein important hai.

## Q44. Service Primitives kya hain?

Exam mein Poocha Gaya: May 2023 Q8i

### 4 Primitives

>> 1. REQUEST

Upper layer lower layer ko kuch karne ki request karta hai.

EXAMPLE: TCP user: CONNECT request karo.

>> 2. INDICATION

Lower layer upper layer ko indicate karta hai ki kuch hua -- doosri side ki request aayi.

EXAMPLE: TCP: CONNECT indication (client aaya hai).

>> 3. RESPONSE

Upper layer lower layer ko INDICATION ka response deta hai.

EXAMPLE: TCP user: CONNECT response (accept/reject).

>> 4. CONFIRM

Lower layer upper layer ko REQUEST ka completion confirm karta hai.

EXAMPLE: TCP: CONNECT confirm (success/fail).

Confirmed Service: Sab 4 primitives use hote hain. Reliable.

Unconfirmed Service: Sirf Request + Indication. No ACK.

## Q45. DNS kya hai?

Exam mein Poocha Gaya: May 2024 Q8iv

### DNS -- Complete

DNS (Domain Name System) internet ka distributed, hierarchical naming system. Human-readable names (www.google.com) ko IP addresses mein translate karta hai. RFC 1034/1035.

### DNS Hierarchy

Root Level (.): Sabse upar. 13 logical root server clusters worldwide.

TLD: .com, .org, .net, .in -- ICANN manage karta hai.

Second Level: google, amazon, flipkart -- organizations register karte hain.

Subdomain: www, mail, ftp -- organization manage karta hai.

### Resolution Process

- Browser cache -> OS cache check.

- Recursive Resolver (ISP's DNS) query karo.

- Root server se .com TLD address.

- .com TLD se google.com authoritative server address.

- Authoritative server se www.google.com IP.

- IP return, browser connect.

UDP port 53 (small). TCP port 53 (large/zone transfers). Caching har level par -- TTL specify karta hai.

## Q46. ALOHA Numerical -- 14.44 kbps, 100-bit frames, Slotted ALOHA. Max throughput?

Exam mein Poocha Gaya: May 2022 Q2b

### Solution

Given: Channel = 14.44 kbps = 14440 bps, Packet = 100 bits, Slotted ALOHA

Slotted ALOHA max throughput = 36.8% of channel

Max throughput = 0.368 * 14440 = 5313.9 bps ~= 5.31 kbps

Or: Frames/sec = 0.368 / (100/14440) = 0.368 / 0.00693 = 53.1 frames/sec

Interpretation: Channel ka 36.8% efficiently use ho sakta hai. Baaki collisions mein waste.

## Q47. Internet Standards aur RFC process.

Exam mein Poocha Gaya: May 2022 Q6a

### Standards Bodies

IETF: Technical standards (protocols). RFC publish karta hai. Open participation.

IANA: IP addresses, port numbers assign karta hai. ICANN ke under.

IEEE: Physical/data link layer standards (Ethernet 802.3, WiFi 802.11).

### RFC Process

- Draft document likhna (Internet-Draft).

- IETF working group review aur discussion.

- IETF Last Call -- community-wide review.

- IESG approval.

- RFC Editor permanent RFC number ke saath publish karta hai.

Famous RFCs: RFC 791 (IP), RFC 793 (TCP), RFC 768 (UDP), RFC 1034/1035 (DNS).

### Benefits

- Vendor-neutral -- koi bhi implement kar sakta hai

- Interoperability -- different systems ek saath kaam karte hain

- Open innovation encouraged

## Q48. Broadband Local Loop technologies.

Exam mein Poocha Gaya: May 2022 Q7b

### DSL (Digital Subscriber Line)

Existing telephone copper par high-speed digital data. Voice simultaneously bhi.

ADSL: Asymmetric. Download 24 Mbps, Upload 3.5 Mbps. Typical home.

VDSL: Short distances (500m). 52 Mbps down. Fiber to cabinet, copper last stretch.

### Cable Modem (HFC)

Cable TV infrastructure. Fiber backbone, coaxial to home. DOCSIS protocol. DOCSIS 3.1 = 10 Gbps theoretical. Shared medium -- peak hours slow.

### FTTH/FTTB (Fiber to Home/Building)

Fiber directly ghar tak. GPON/EPON. Speeds: 1 Gbps symmetric. Future: 10-40 Gbps. Jio GigaFiber example.

### Fixed Wireless Access

WiMAX, LTE fixed wireless, 5G FWA. Rural areas ke liye best.

### Satellite Broadband

GEO: High latency 270ms. LEO (Starlink): ~20ms, 150-500 Mbps. Remote/rural areas.

SABHI 5 PAPERS KE SABHI QUESTIONS COVER HO GAYE!

May 2022 | May 2023 | May 2024 | Dec 2024 | Jun 2025

### Exam Preparation Tips

- OSI Model -- har layer ka naam, kaam, protocol yaad karo. Mnemonic use karo.

- TCP vs UDP -- difference clearly, header fields kyun hain.

- GBN vs SR -- window formulas: GBN=2^n-1, SR=2^(n-1). Numerical aayenge.

- ALOHA -- throughput: Pure=18.4%, Slotted=36.8%. Derivation samjho.

- Subnetting -- step-by-step practice karo.

- TCP 3-way handshake -- diagram ke saath states yaad rakho.

- TCP Congestion Control -- Slow Start, Congestion Avoidance, Fast Retransmit.

- Hamming Code -- encoding step-by-step practice karo.

- Shannon's formula -- C = B*log2(1+SNR). Numerical mein zaroor aata hai.

- Short notes: ICMP, ARP/RARP, DNS, SNMP, FDDI, FSM -- 2-3 paragraphs.

BEST OF LUCK! Mehnat rang laayegi!